# Programming Reader

- Open Transport Tray: Start>All Programs>BEST Access>BEST  Transport

- Your panels and readers should show

up in the program. Locate the 
reader you would like to program

and right click on it and click on

Configure Lockset.

# Downloading Single Reader

- Click on the system tree icon to 
expand the hardware tree.

- Locate the Offline panel the 
reader you would like to 
download is attached to 
and click on the right arrow to expand the hardware tree.

- Right click on the reader you would like to download and click the download option.

- Sync completed successfully once the reader has successfully downloaded.

- Close the programs, system administration, BASIS Com Server, and BASIS Transport tray.

# Downloading Entire Panel

- Open System Administration, Start>All Programs>OnGuard>System Administration

- Open Access Panels: AccessControl>Access Panels

- Locate the Offline tab and click on it to display offline lock access panels.

- Right click on the access panel to download the offline panels. Select the Download option.

- You will be prompted that this will force a full system download. Click on the OK button.

- Through the download process you will see notifications from the BEST Transport Tray in the lower right corner by your clock. Once the download is completed, you will see a Sync Completed Successfully message.

# Close Programs

- Expand system tray by clicking on the 
up arrow and right click on the COM 
Server to exit that program. Complete 
the same steps with the BASIS 
Transport Tray.

- Close system administration.

# Downloading Using BEST Transport Tray

- Login to System Administration and make programming changes to users, access levels, time zones, etc.

- Once programming changes have been completed, you are ready to download your readers for programming.

- To download your offline readers, you will open the BASIS Communication Server by navigating to Start>Programs>OnGuard> Services and Support> Communication Server.

Figure 1: The communication server will open and run in the lower right corner of your screen.

- Open Transport Tray Start>Programs>Best Access> BEST Transport>BEST Transport Tray

- Both the OnGuard COM Server and the BEST Transport Tray Should be running in your system tray.

# Using BEST Transport and Transport Tray on the OnGuard Computer